# LoRaWAN System Data Flow & Gateway Aggregation Protocol Document

## 1. Overview

This document defines the end-to-end protocol for data flow in a LoRaWAN-based system, starting from end devices (nodes), passing through LoRa gateways, network server (ChirpStack), and finally reaching MQTT and cloud applications. It also defines how gateway-level aggregation of multiple node data can be implemented for IoT applications such as aquaculture monitoring systems.

## 2. System Components

### 2.1 End Devices (LoRa Nodes)

- STM32-based sensor nodes (or similar MCU)

- Collect sensor data (temperature, pH, battery, etc.)

- Send uplink packets using LoRaWAN stack

### 2.2 LoRa Gateway

- Receives RF LoRa signals

- Forwards raw packets to network server

- Does NOT decrypt or interpret payload

### 2.3 Network Server (ChirpStack)

- Validates MIC and frame counters

- Manages device sessions

- Handles ADR (Adaptive Data Rate)

- Routes data to Application Server

### 2.4 Application Server (ChirpStack)

- Decrypts payload using AppSKey

- Decodes payload using codec

- Publishes structured JSON to MQTT

### 2.5 MQTT Broker

- Message distribution layer

- Used by cloud applications (AWS IoT, backend services)

### 2.6 Cloud Application Layer

- AWS IoT Core / Lambda / DB / Dashboard

- Stores and processes IoT data

## 3. End Device Data Format

### 3.1 Application Payload Structure (Example)

Each node sends binary payload:

[DeviceID (2 bytes)] [Temperature (2 bytes)] [Humidity (2 bytes)] [Battery (2 bytes)] [Counter (4 bytes)]

### 3.2 STM32 Example Encoding

uint8_t payload[12];

payload[0] = 0x01; payload[1] = 0x23; // Device ID
payload[2] = 0x00; payload[3] = 0x1E; // Temperature
payload[4] = 0x00; payload[5] = 0x55; // Humidity
payload[6] = 0x03; payload[7] = 0xE8; // Battery
payload[8] = 0x00; payload[9] = 0x00; payload[10] = 0x00; payload[11] = 0x10;

AppData.BufferSize = 12;
memcpy(AppData.Buffer, payload, 12);

## 4. LoRaWAN Uplink Flow

### Step 1: Node Transmission

- MCU sends raw bytes via LoRaWAN stack

- Payload is encrypted using LoRaWAN session keys

### Step 2: Gateway Reception

- Gateway receives RF signal

- Converts to PHY payload

- Forwards to network server

### Step 3: Network Server Processing

- Validates packet integrity

- Extracts DevAddr

- Maps to registered device

### Step 4: Application Server Processing

- Decrypts FRMPayload using AppSKey

- Applies payload codec (decoder)

- Produces structured JSON

### Step 5: MQTT Publication

- Publishes decoded message to topic:

application/<app_id>/device/<dev_eui>/event/up

## 5. MQTT Message Structure (Decoded Output)

### Example Payload

{
  "deviceInfo": {
    "devEui": "A84041F0...",
    "deviceName": "Node-1"
  },
  "rxInfo": {
    "gatewayId": "8aa29efffe89577d"
  },
  "decodedPayload": {
    "deviceId": 291,
    "temperature": 30,
    "humidity": 85,
    "battery": 1000
  }
}

## 6. Gateway Aggregation Requirement

### 6.1 Problem Statement

A single gateway receives uplinks from multiple nodes. Raw forwarding sends individual packets per node.

Requirement:

- Aggregate multiple node data before sending to cloud OR processing layer

## 7. Gateway Aggregation Model (Design)

### 7.1 Logical Aggregation Layer (Recommended)

Instead of modifying LoRaWAN stack:

Gateway → ChirpStack → MQTT → Aggregator Service → Cloud

### 7.2 Aggregation Service Responsibilities

- Subscribe to MQTT topic

- Maintain in-memory buffer per gateway

- Group node data within time window

- Publish aggregated packet

### 7.3 Example Aggregation Logic

#### Input messages:

Node A → temp=30
Node B → temp=28
Node C → temp=31

#### Aggregated output (per 10 sec window):

{
  "gatewayId": "8aa29efffe89577d",
  "timestamp": 1710000000,
  "nodes": [
    {"devEui": "A1", "temp": 30},
    {"devEui": "B2", "temp": 28},
    {"devEui": "C3", "temp": 31}
  ]
}

## 8. Recommended Architecture

### 8.1 DO NOT aggregate at gateway firmware level

Reasons:

- LoRaWAN is asynchronous

- Different SF/DR causes timing mismatch

- Gateway is stateless by design

### 8.2 DO aggregation at cloud/middleware layer

Best options:

- Node.js MQTT aggregator

- AWS Lambda stream processor

- Kafka stream processor

## 8.3 Gateway-side Edge Aggregator (Optional Node.js on Raspberry Pi)

### Overview

In advanced deployments (aquaculture farms, industrial IoT, remote clusters), a Raspberry Pi or edge gateway can run a Node.js Edge Aggregator Service. This service processes MQTT data locally AFTER ChirpStack decoding.

It enhances reliability, reduces cloud load, and enables offline-first intelligence.

### Architecture

LoRa Nodes → Gateway → ChirpStack → MQTT Broker → Edge Aggregator (Raspberry Pi)
                                               ↓
                                      AWS IoT / Cloud / DB

## 8.3.1 Core Responsibilities

- Subscribe to ChirpStack MQTT application topics

- Buffer multi-node data in real-time

- Perform time-window aggregation (5s–60s configurable)

- Detect anomalies locally (optional rule engine)

- Forward processed data to AWS IoT Core or cloud broker

## 8.3.2 Production-Grade Buffer Layer (Redis-based)

### Why Redis instead of memory?

- Prevents data loss on crash

- Supports multi-process scaling

- Enables persistence + recovery

- Handles high-frequency IoT streams

### Redis Data Model

Key: gateway:{gatewayId}:node:{devEui}
Value: JSON array of sensor readings
TTL: 60–300 seconds

### Example Redis Integration (Node.js)

import mqtt from "mqtt";
import Redis from "ioredis";

const redis = new Redis();
const client = mqtt.connect("mqtt://localhost:1883");

client.on("message", async (topic, msg) => {
  const data = JSON.parse(msg.toString());

  const devEui = data.deviceInfo?.devEui;
  const gatewayId = data.rxInfo?.[0]?.gatewayId;

  const key = `gateway:${gatewayId}:node:${devEui}`;

  await redis.rpush(key, JSON.stringify(data.decodedPayload));
  await redis.expire(key, 120);
});

## 8.3.3 Auto-Reconnect + QoS Handling

### MQTT Reliability Strategy

- Use QoS 1 for guaranteed delivery

- Enable persistent session

- Auto reconnect with exponential backoff

### Example MQTT Config

const client = mqtt.connect("mqtt://localhost:1883", {
  reconnectPeriod: 2000,
  clean: false,
  keepalive: 60
});

### QoS Rules

| Level | Meaning |
| --- | --- |
| QoS 0 | Fast, no guarantee |
| QoS 1 | At least once (recommended) |
| QoS 2 | Exactly once (rare, heavy) |

## 8.3.4 Time Window Aggregation Engine

setInterval(async () => {
  const keys = await redis.keys("gateway:*");

  for (const key of keys) {
    const data = await redis.lrange(key, 0, -1);

    const aggregated = {
      gatewayId: key.split(":")[1],
      timestamp: Date.now(),
      samples: data.map(d => JSON.parse(d))
    };

    await publishToCloud(aggregated);
    await redis.del(key);
  }
}, 10000);

## 8.3.5 Data Persistence Layer

### Option A: SQLite (Lightweight Edge Storage)

- Local fallback database

- Stores last N hours of data

CREATE TABLE sensor_data (
  id INTEGER PRIMARY KEY AUTOINCREMENT,
  devEui TEXT,
  gatewayId TEXT,
  payload TEXT,
  timestamp DATETIME DEFAULT CURRENT_TIMESTAMP
);

### Option B: InfluxDB (Recommended for IoT)

- Time-series optimized

- Great for dashboards

- Works with Grafana

Measurement:

loRa_uplink

Fields:

- temperature

- humidity

- battery Tags:

- devEui

- gatewayId

## 8.3.6 AWS IoT Core Forwarding

### Connection Options

- MQTT over TLS (preferred)

- X.509 certificate authentication

### AWS IoT MQTT Publish

import awsIot from "aws-iot-device-sdk";

const device = awsIot.device({
  keyPath: "cert/private.pem.key",
  certPath: "cert/certificate.pem.crt",
  caPath: "cert/AmazonRootCA1.pem",
  clientId: "edge-gateway-1",
  host: "xxxxx-ats.iot.ap-south-1.amazonaws.com"
});

function publishToCloud(data) {
  device.publish("aquafarm/aggregated", JSON.stringify(data), { qos: 1 });
}

## 8.3.7 Docker Deployment (Raspberry Pi)

### Dockerfile

FROM node:18-alpine

WORKDIR /app

COPY package*.json ./
RUN npm install

COPY . .

CMD ["node", "index.js"]

### Docker Compose

version: "3"

services:
  aggregator:
    build: .
    restart: always
    network_mode: "host"
    environment:
      - MQTT_URL=mqtt://localhost:1883
      - REDIS_URL=redis://localhost:6379

## 8.3.8 Production Edge Scenarios (Often Missed)

### 1. Network Drop Handling

- Buffer locally in Redis

- Retry cloud sync

### 2. Duplicate Packet Handling

- Use (devEui + fCnt) deduplication

### 3. Clock Drift on Raspberry Pi

- Use NTP sync mandatory

### 4. Power Failure Recovery

- Redis AOF persistence enabled

- SQLite backup sync

### 5. Burst Traffic Handling

- Queue-based ingestion

- Rate limiting per node

### 6. Multi-Gateway Conflict

- Deduplicate same uplink from multiple gateways

### 7. Memory Pressure

- TTL-based key expiration

- Batch flush mechanism

## 8.3.9 Summary

Edge aggregation is a performance and resilience layer, not a replacement for ChirpStack. It is used to:

- Optimize bandwidth

- Enable local intelligence

- Support offline operation

- Reduce cloud dependency

## 9. Data Flow Summary Data Flow Summary

[STM32 Nodes]
    ↓ LoRaWAN Uplink (Encrypted)
[Gateway]
    ↓ Forward PHY Payload
[ChirpStack Network Server]
    ↓ Validate + Route
[ChirpStack App Server]
    ↓ Decrypt + Decode
[MQTT Broker]
    ↓ Publish JSON
[Aggregator Service]
    ↓ Combine node data
[Cloud / Dashboard]

## 10. Key Rules

- Gateway MUST NOT modify payload

- Decryption happens ONLY in Application Server

- MQTT carries decoded or raw depending on topic

- Device identity comes from DevEUI mapping

- Aggregation should be external service

## 11. Conclusion

This architecture ensures scalability, security, and proper LoRaWAN compliance while enabling multi-node aggregation for industrial IoT applications such as aquaculture systems.